Voice Procedures

Telecommunication Unit
I.C.S.S.

Procedure for Radio Communication (Voice)

1.	Check that the Radio Set is prepared for operation.

a.	VHF Hand-Helds – always ensure that the battery is charged and have a spare available. Check antenna is attached.

b.	HF Mobiles/Base. Check power supplies regularly. Check cable connections i.e. antenna, power supply, microphone.

2.	Transmitting.

a.	Decide beforehand what you are going to say, ensuring that it will be clear and concise. Make notes if necessary.

b.	Check that no-one else is speaking on the net before you start. Do not attempt to use Selcall (HF Mobiles/Base) if the net is busy.

c.	Avoid excessive calling. In particular with Selcall, allow time for the remote station's automatic reply. (This can take up to 40 secs). Calling again too soon after the original call may corrupt an incoming reply from the called station. The called station may be able to hear you but be unable to reply, or you may not hear their reply. Make one last call stating NIL HEARD – OUT. The other station can then call you back when they are in a better position to do so.

d.	If your message is long, divide it into sensible phrases, pause to allow your message to be understood (and written down if necessary) while maintaining a natural rhythm.

e.	Use standard pronunciation. Emphasize vowels sufficiently. Avoid extremes of pitch, do not let your voice drop at the end of a sentence. Speak in a moderately strong voice but do not shout as this will distort the signal.

f.	Hold the microphone at about 5cms from your lips while speaking. Shield the microphone from background noises.

g.	Be aware that everything that you say can be heard by anyone using a radio receiver on the same frequency. Depending on the situation in the country of operations, it may be wise to use simple codes (e.g. alphanumeric codes for locations, initials for names etc.).

h.	Ensure that each user on the network is registered on a master list held by the Radio Room, and that any unauthorized callsigns heard on the net are noted and the users (if known) are properly registered or informed that they do not have permission to use the network.

i.	Never transmit military or other confidential information on UN networks. Doing so could seriously jeopardize telecommunications and may result in the withdrawal of permission to operate, or confiscation of, the station by the competent authorities.

Example of Radio Conversation

The calling station is indicated by the use of THIS IS or FROM.

Juliet Hotel this is Romeo Foxtrot – message – over

This is Juliet Hotel – send – over

From Romeo Foxtrot – Convoy 5a has just passed Sierra 3. ETA Sierra 4 at 14 30 hours – over

Juliet Hotel – Roger – out.

Radio Checks

Radio checks should be carried out periodically during periods of low traffic.

Signal quality is reported as STRENGTH/READABILITY as follows:

Signal strength

LOUD	Your signal is strong.
GOOD	Your signal is good.
WEAK	I can hear you but with difficulty.
VERY WEAK	I can hear you but with great difficulty.
NOTHING HEARD	I cannot hear you at all.

Readability

CLEAR	Excellent quality
READABLE 	Good quality. No difficulty in reading you.
DISTORTED	I have problems reading you.
WITH INTERFERENCE	I have problems reading you due to interference.
NOT READABLE	I can hear that you are transmitting but cannot read 
	you at all.

e.g.

Three Zero One – this is Two Five Nine – Radio Check – over. 

From Three Zero One – (you are) loud and clear – over. 

This is Two Five Nine – (you are) good and readable – out.

Letter	      Phonetic Equivalent	Numeral	Spoken As

A	Alpha	0	Zero
  B	Bravo	1	Wun
  C	Charlie	2	Too
  D	Delta	3	Thuh–ree
  E	Echo	4	Fo–wer
  F	Foxtrot	5	Fife
  G	Golf	6	Six
  H	Hotel	7	Seven
  I	India	8	Ate
  J	Juliet	9	Niner
  K	Kilo
  L	Lima
  M	Mike	In general, numbers are
  N	November	transmitted digit by digit
  O	Oscar	except that exact multiples
  P	Papa	of hundreds and thousands
  Q	Quebec	are sent as such.
  R	Romeo	
  S	Sierra	e.g.
  T	Tango	12	Twelve
  U	Uniform	44	Fo–wer Fo–wer
  V	Victor	90	Niner Zero
  W	Whisky	136	Wun Thuh–ree Six
  X	X-Ray	500	Fife Hundred
  Y	Yankee	16000	Wun Six Thousand
  Z	Zulu

Procedure Words

ProWord 	Meaning

ACKNOWLEDGE 	Confirm that you have received message and will comply 		(WILCO)

AFFIRMATIVE 	Yes/Correct

NEGATIVE 	No/Incorrect

ALL AFTER 	Everything that you (I) transmitted after ........
	
ALL BEFORE 	Everything that you (I) transmitted before ........

BREAK – 	All stations will immediately cease transmission on  
BREAK –	hearing this ProWord. The station BREAKing has urgent  BREAK!	life-saving message. Only to be used in EXTREME 		emergency.

CORRECT	You are correct

CORRECTION	The correct version is .........

WRONG	Your last transmission was incorrect. The correct version
	is ..........

DISREGARD THIS	This transmission is an error. Disregard it.
TRANSMISSION – 	
OUT

DO NOT ANSWER – 	Station(s) called are not to answer this call,
OUT	acknowledge this message, or to transmit in
	connection with this transmission

FIGURES	Numbers follow (in message)

MESSAGE	I have an informal message for you

MESSAGE FOLLOWS	I have a formal message which should be recorded 
	(e.g. written down).

SILENCE –	Cease all transmission immediately.
SILENCE –	Will be maintained until lifted.
SILENCE!	(Used by Network Control Operator).

SILENCE LIFTED	Silence is lifted. Net is free for traffic.

SPEAK SLOWER	Adjust the speed of your transmission.
      (FASTER)

I SPELL	I shall spell the next word phonetically.

THROUGH ME	I am in contact with the station you are calling.
	I can act as a relay station.

MESSAGE 	
PASSED TO ........	Your message has been passed to ......

UNKNOWN STATION	The identity of the station calling or with whom I am
	attempting to establish communication is unknown.

VERIFY	Verify entire message (or portion indicated) with the
	originator and send correct version. To be used only at
	discretion or by the addressee to which the questioned
	message was sent.

I VERIFY	That which follows has been verified at your
	request and is repeated. To be used only as a
	reply to VERIFY.

WAIT (WAIT – WAIT)	I must pause for a few seconds.

WAIT – OUT	I must pause for longer than some seconds and
	will call you again when ready.

WILCO	I have received and understood your message
	and will comply. (Only used by the addressee).
	
	ROGER and WILCO are never used together.

OVER	I have finished my turn – a response is expected.
	Go ahead, transmit.

OUT	I have finished my transmission. No reply is
	expected.

	(OVER and OUT are never used together.)

OUT TO YOU	I have nothing more for you, do not reply. I
	shall now call another station on the net.

READ BACK	Read back the following message to me exactly
	as received.

I READ BACK	The following is my reply to your request to
	read back.

RELAY TO .........	Transmit the following message to all addressees
	or to the address immediately following.

RELAY THROUGH	Send this message by way of callsign .........

ROGER	I have received your last transmission satisfactorily.

ROGER SO FAR?	Have you received this part of my message
	satisfactorily?

SAY AGAIN	Repeat all of your last transmission.

SAY AGAIN	Repeat portion of message indicated.
ALL (WORD)
AFTER (BEFORE)

I SAY AGAIN	I am repeating my transmission or portion
	indicated.

SEND	Go ahead with your transmission.

SEND YOUR 	Go ahead, I am ready to copy.
MESSAGE

WORD AFTER .........	The word of the message to which I refer is
	that which follows .............

WORD BEFORE ...........	The word of the message to which I refer is
	that which follows .............

WORDS TWICE	Communication is difficult. Transmitting each 
	phrase twice. This proword can be used as an
	order, request or information.