| SPARC Enterprise Mx000 Servers XSCF User’s Guide |  |
| --- | --- |

C H A P T E R 5

Overview of the XSCF Shell

This chapter describes how to use the XSCF Shell. The chapter also describes how to use commands and log in with an XSCF user account. It also explains command errors.

## 5.1 Overview of the XSCF Command Shell

Performing certain XSCF commands on the XSCF Shell terminal can display the server status so that control and configuration information related to system operation can be viewed.

The XSCF commands are effectively used by users who have created user accounts for the XSCF Shell terminal but cannot use the XSCF Web.

The following XSCF commands can be used from the XSCF Shell prompt (XSCF>) displayed after login to XSCF.

There are nine user privilege types as described below. For details on setting the user privilege, see Chapter 2. For details on the operations available with each user privilege level, see Chapter 1 or the Administration Guide.

- domainop: The user can refer to all status information in a domain.

- domainmgr: The user can perform domain power operations.

- domainadm: The user has domain administrator privilege and can perform every type of domain operation.

- platop: The user can refer to all status information in the entire system.

- platadm: The user has system administrator privilege and can use almost all XSCF Shell commands

- useradm: The user can manage user accounts.

- auditop: The user can refer to the audit method of the XSCF and the audit records.

- auditadm: The user can control the audit to the XSCF.

- fieldeng: The user can perform the commands for FEs.

TABLE 5-1 outlines the XSCF Shell commands. For details on each command and user privileges, see the man page or the XSCF Reference Manual.

| TABLE 5-1 XSCF Commands |  |
| --- | --- |
| Command | Description |
| adduser | Creates an XSCF user account. |
| deleteuser | Deletes an XSCF user account. |
| disableuser | Disables an XSCF user account |
| enableuser | Enables an XSCF user account. |
| showuser | Displays XSCF user account information. Alternatively, it displays the user's own user account information. |
| password | Changes an XSCF user account password. Alternatively, it changes the user's own XSCF user account password. |
| setpasswordpolicy | Configures the XSCF password policy. |
| showpasswordpolicy | Displays the XSCF password policy. |
| setprivileges | Assigns user privileges. |
| setnetwork | Configures the XSCF network. |
| shownetwork | Displays the XSCF network settings and XSCF-LAN network status. |
| setroute | Configures the XSCF-LAN route. |
| showroute | Displays XSCF-LAN route settings. |
| sethostname | Specifies the XSCF-LAN host name and domain name. |
| showhostname | Displays the XSCF-LAN host name and domain name. |
| setnameserver | Sets a name server (DNS) with XSCF. |
| shownameserver | Displays the XSCF name server(s). |
| applynetwork | Applies the network settings. |
| setdscp | Configures DSCP. |
| showdscp | Displays the DSCP settings. |
| nslookup | Checks for the name resolution of a host name. |
| settimezone | Specifies the time zone. |
| showtimezone | Displays the time zone setting. |
| setdate | Sets the XSCF time. |
| showdate | Displays the XSCF time. |
| setntp | Configures the NTP server. |
| showntp | Displays the SSH settings and fingerprint. |
| resetdateoffset | Resets the time subtraction between the XSCF and the domain. |
| setssh | Configures SSH. Generates RSA and DSA keys for SSH2 host authentication. |
| showssh | Displays the SSH settings and fingerprint. |
| settelnet | Configures telnet. |
| showtelnet | Displays the telnet settings. |
| setautologout | Sets the session timeout time of the XSCF shell. |
| showautologout | Displays the session timeout time of the XSCF shell. |
| setsmtp | Configures the SMTP server. |
| showsmtp | Displays the the SMTP server settings. |
| setemailreport | Configures mail notification. |
| showemailreport | Displays the mail notification settings. |
| setlookup | Enables or disables the use of an LDAP server for authentication and user privilege lookup. |
| showlookup | Displays information about whether an LDAP server is used for authentication and user privilege lookup. |
| setldap | Configures LDAP client settings. |
| showldap | Displays LDAP client settings. |
| sethttps | Configures the https settings. |
| showhttps | Displays the https settings. |
| setupplatform | - Sets up platform-specific settings. Note: In the platform-specific settings, the following items can be optionally configured. Each item is the same as the setting of Chapter 2. - User Account settings - XSCF Network Settings - Internal DSCP Network - Domain Name Service - Network Time Protocol - SSH - HTTPS Server - Email Reports |
| setarchiving | Configures the log archiving function of XSCF. |
| showarchiving | Displays the settings of the log archiving function of XSCF. |
| setaudit | Configures the auditing of XSCF. |
| showaudit | Displays the settings of the audit of XSCF. |
| viewaudit | Displays the audit records (Audit trail) of XSCF. |
| setsnmp | Configures the SNMP Agent. |
| showsnmp | Displays the SNMP Agent settings. |
| setsnmpusm | Configures the USM management information for the SNMP agent. |
| showsnmpusm | Displays the USM management information for the SNMP agent. |
| setsnmpvacm | Configures the VACM management information for the SNMP agent. |
| showsnmpvacm | Displays the VACM management information for the SNMP agent. |
| setdcl | Specifies the domain configuration information (DCL). |
| showdcl | Displays the domain configuration information. |
| showboards | Displays the component information and the COD information about a system board. |
| showdevices | Displays the domain information specified for a system board. |
| showdomainstatus | Displays the domain status. |
| setupfru | Specifies the number of XSB partitions of the system board and sets the memory mirror mode. |
| showfru | Displays the specified number of XSB partitions of the system board and the memory mirror mode that is set. |
| testsb | Diagnoses the system board. |
| addboard | Adds a system board to a domain. |
| deleteboard | Deletes a system board from a domain. Alternatively, it reserves a delete to the system board. If the deletion is reserved, the deletion is done after OS reboot. |
| moveboard | Moves a system board to another domain. Alternatively, it reserves a move to the system board. If the movement is reserved, the deletion is done after OS reboot of source domain, and the assignment to the new domain is done. |
| replacefru | Replaces a FAN unit, PSU, panel, system board, XSCF Unit, or IO-BOX in work performed according to the applicable guide. |
| addfru | Installs a FAN unit, PSU, system board, or IO-BOX in work performed according to the applicable guide. |
| deletefru | Removes a FAN unit, PSU, system board, or IO-BOX in work performed according to the applicable guide. |
| setdomainmode | Sets a hardware initial diagnostic level (No, standard, maximum). Enables or disables break signal sending, host watchdog, and automatic boot. |
| showdomainmode | Displays the hardware initial diagnostic level. Displays information of enabled or disabled status on break signal sending, Host watchdog , and automatic boot. |
| sendbreak | Sends a break signal to the server. |
| showresult | Displays the exit status of the most recently executed command. |
| setlocale | Sets locale. |
| showlocale | Displays locale. |
| setaltitude | Sets altitude. |
| showaltitude | Displays altitude. |
| cfgdevice | Sets the connection destination of the DVD drive unit and tape drive unit. Displays the setting status information. |
| addcodlicense | Applies the license key obtained from the license center to the system. |
| deletecodlicense | Deletes the license key applied to the system. |
| showcodlicense | Displays the license keys applied to the system. |
| showcodusage | Displays license usage information. |
| setcod | Configures COD settings. |
| showcod | Displays COD settings. |
| console | Connects to a domain console. |
| showconsolepath | Displays the operating status of the main console. |
| showenvironment | Displays the temperature, humidity, voltage, and fan rotation speed. |
| showstatus | Lists degraded components. |
| showhardconf | Displays all components mounted in the server. |
| poweron | Turns on power to all domains or the specified domain. |
| poweroff | Turns off power to all domains or the specified domain. |
| reset | Resets the specified domain. Note: The following three reset modes are available: por: Domain system reset request: Domain panic instruction xir: Domain CPU reset |
| setpowerupdelay | Sets the warm-up time and the air-conditioning wait time. |
| showpowerupdelay | Displays the warm-up time and the air-conditioning wait time settings. |
| setshutdowndelay | Sets the UPS shutdown delay time at power failure. |
| showshutdowndelay | Displays the UPS shutdown delay time at power failure. |
| setdualpowerfeed | Sets the dual power feed. |
| showdualpowerfeed | Displays the dual power feed. |
| setlocator | Enables or disables the CHECK LED blinking. |
| showlocator | Displays the LED status. |
| switchscf | Switches the XSCF Unit state (Active/Standby). |
| ioxadm | Configures the External I/O Expansion Unit (IOBOX). Displays the External I/O Expansion Unit settings. |
| clockboard | Sets/Displays the number of clock unit (CLKU) used when the next platform is powered on. |
| setdomparam | Rewrites the OpenBoot PROM environment variable that is compulsory. |
| getflashimage | Gets the firmware update program. |
| flashupdate | Updates the firmware program. |
| version | Displays the comprehensive firmware (XCP) version. Displays the XSCF firmware version. Displays the OpenBoot PROM firmware version. |
| prtfru | Displays the FRU-ROM data. |
| snapshot | Saves log information to the specified destination. |
| showmonitorlog | Displays the XSCF monitoring messages on console in real time. |
| showlogs | Displays an error log, power log, event log, console log, panic log, IPL log, temperature/humidity log, and monitoring message log. |
| fmadm | Monitors or controls the Fault Management Diagnosis Engines (FMDE). |
| fmdump | Dumps the fault event log containing FM diagnosis results. |
| fmstat | Displays the FMDE status. |
| unlockmaintenance | Forcibly release the locked status of the XSCF. |
| rebootxscf | Resets the XSCF. |
| who | Displays users who login to the XSCF. |
| man | Displays the man page of the specified command. |
| exit | Ends the XSCF Shell. |

## 5.2 Login to XSCF User Accounts

This section describes how to log in to XSCF. The user can log in to XSCF from an XSCF-LAN port using either SSH or telnet, or from the serial port.

### 5.2.1 Before Logging In

Note the following before attempting to log in:

- For details on how to create, add, and delete user accounts, see Chapter 2.

- After login, if the shell has not been accessed for a certain period, XSCF automatically terminates the shell. The default timeout period is 10 minutes. The timeout period can be specified. For details on specifying the timeout period, see Chapter 2.

- In one domain, only one user can use the RW console (write-enabled console). While one user is using the RW console, another user cannot start another RW console in the same domain. For details on console connection, see Chapter 3.

- When a login fails, see Appendix D.

Note - In this manual, the window of the XSCF Shell terminal is called the XSCF console.

### 5.2.2 Operation From a Terminal Connected to the Serial Port

This section describes how to log in from a terminal connected to the serial port.

1. After the terminal is connected to the serial port, press return on the terminal.

2. Enter a user account and password when prompted by XSCF.

3. Enter XSCF commands at the shell prompt (XSCF>) that is displayed after login to XSCF.

The following is a login example:

login: jsmith Password: xxxxxxxx XSCF>

### 5.2.3 Operation for Connecting Via the XSCF-LAN (SSH)

This section describes how to log in to XSCF through an XSCF-LAN (SSH) connection.

1. Before logging in using SSH, check that the fingerprint is pre-stored. If you did not save the fingerprint, please connect to the serial port and use showssh (8) to make a memo of the fingerprint of the host public key.

2. From an SSH client, specify the IP address or host name of XSCF and the port number, if necessary (default port number 22), and use SSH connection via XSCF-LAN.

3. Enter a user account and password when prompted by XSCF.

4. The fingerprint of the host public key may be displayed with a request to confirm its authenticity. If it is authentic, enter "yes" as the response, since the XSCF is correct with confirming the fingerprint.

5. Enter XSCF commands at the shell prompt (XSCF>) that is displayed after log in to XSCF.

Note - To set the password for an XSCF user account, use the password (8) command. The shownetwork (8) command can be used to display XSCF-LAN setting information and the current network connection status.

The following is a login example:

[foo@phar foo]% ssh june@192.168.0.2 The authenticity of host '192.168.0.2 (192.168.0.2)' can't be established. RSA key fingerprint is 03:4b:b4:b2:3d:4d:0c:24:03:ca:f1:63:f2:a7:f3:35. Are you sure you want to continue connecting ? [yes¦no] : yes Warning: Permanently added '192.168.0.2' (RSA) to the list of known hosts. foo@phar's password:xxxxxx XSCF>

When the SSH connection is done using a user key, install the user public key in XSCF in advance. See Chapter 2 for instructions on how to install the user public key.

The following example shows a login using a user public key:

[client]# ssh nana@192.168.1.12 Enter passphrase for key ‘/home/nana/.ssh/id_rsa’: xxxxxxx Warning: No xauth data; using fake authentication data for X11 forwarding. Last login: Mon Sep 1 10:19:37 2006 from client XSCF>

### 5.2.4 Operation For Connecting Via the XSCF-LAN (Telnet)

This section describes how to log in to XSCF via an XSCF-LAN (telnet) connection.

1. Enter the IP address or host name of XSCF and port number 23, and use telnet via XSCF-LAN.

2. Enter a user account and password from the XSCF console.

3. Enter XSCF commands at the shell prompt (XSCF>) that is displayed after you login to XSCF.

4. The following is a login example:

login:jsmith Password:xxxxxxxx XSCF>

## 5.3 View Server Status and Control Commands

This section describes the typical XSCF Shell commands that can be used to display the server status, operate the server, and control the server. For details on the commands, see the man page or the XSCF Reference Manual.

- showenvironment

- showlocator/setlocator

- showconsolepath

- fmadm / fmdump / fmstat

- showdomainstatus

- reset / poweron / poweroff

- sendbreak

##### showenvironment

The showenvironment (8) command displays the values of all sensors in the server. By finding out the intake temperature, humidity, voltage, and fan rotation speed in the server, the system administrator can check for errors in the system environment.

##### showlocator/setlocator

These commands display status information indicated by the LEDs on devices and the operator panel of the server. Finding out information on device errors is helpful in component degradation and replacement. Also, the system administrator can use the commands to identify the target device among many devices.

##### showconsolepath

The showconsolepath (8) command displays the operating status of the domain console. By finding out the users of domain consoles, the system administrator can notify the users before a user performs a server operation or server control.

##### fmadm / fmdump / fmstat

The server has an architecture that performs fault management (FMA) for CPUs, memory, and the I/O system during OS running. The system administrator can use the fmadm (8) command to display configuration and status information about individual FMA modules that detect faults, perform fault diagnoses, and resolve faults. The command can also list faulty and degraded resources. The fmstat (8) command displays the processing time and number of events for each FMA module. The fmdump (8) command displays detailed fault information so that system administrator can determine faulty resources.

##### showdomainstatus

The showdomainstatus (8) command displays the current operating status of a domain. The system administrator can find out the status of each domain from its power on time to its operation start time.

##### reset / poweron / poweroff

There are three types of resets: the system reset, the panic instruction, and the CPU reset. To reset a domain, the system administrator can perform the reset (8) command with one of these three types specified. Performing the poweron (8) or poweroff (8) command can turn power on or off to a constructed domain in the system configuration.

##### sendbreak

The system administrator can use the sendbreak (8) command to send a break signal to the OS.

## 5.4 Server Configuration Information Commands

This section describes the typical XSCF Shell commands used to display configuration information on components in the server, such as the number of CPUs and memory capacity, the XSCF network configuration, the time, and degradation information.

- showhardconf

- shownetwork / showroute / shownameserver / showdscp

- showntp / showdate

- showstatus

##### showhardconf

The showhardconf (8) command lists all the components mounted in the server and their status information. A problem component is indicated by a mark (*). The system administrator can check the component configurations and the numbers of different types of components.

##### shownetwork / showroute / shownameserver / showdscp

The shownetwork (8) command displays the IP addresses, masks, and network connection information for the XSCF-LAN and ISN installed in the XSCF Unit. Also, the shownetwork (8) command displays the XSCF network connection status. By finding out the amount of data sent or received through a particular interface, the system administrator can check the LAN connection status and the management network load. The showroute (8) command displays routing environment such as destination IP addresses. The shownameserver (8) displays the DNS server. The system administrator can view the interface information required for the XSCF network. The showdscp (8) command displays the IP addresses assigned for DSCP usage.

##### showntp / showdate

The showntp (8) command displays the NTP server configured with the server. The showdate (8) command displays the system standard time (XSCF time). The system administrator can use the showdate (8) command to determine the reference time used in the server.

##### showstatus

The system administrator can use the showstatus (8) command to list degraded components.

## 5.5 Domain Control and Maintenance Commands

This section describes the typical XSCF Shell commands that manage resource assignment to domains and resource removal from domains, install devices, remove devices, replace devices, and enable or disable functions.

- showdevices / cfgdevice

- console

- showdcl / setdcl

- showfru / setupfru

- addfru / deletefru / replacefru

- showboards / addboard / deleteboard / moveboard

- showdomainmode / setdomainmode

##### showdevices / cfgdevice

The cfgdevice (8) command displays the domain to which a DVD drive unit or tape drive unit is assigned. The showdevices (8) command displays the operating status of resources installed on a system board (XSB). The system administrator can use this command to determine the devices to be assigned to a domain and check whether the DR function can be used to connect or disconnect an XSB.

##### console

The console (8) command establishes a connection to the domain console. This command supports both interactive and read-only connections.

##### showdcl / setdcl

The showdcl (8) command displays the domain configuration information (DCL) specified for individual domains or LSBs that compose a domain, and the setdcl (8) command specifies the configuration. The system administrator refers to and specifies DCL when adding an XSB to a domain.

##### showfru / setupfru

The showfru (8) command displays the locations of devices, such as system boards, mounted in the server and resource partition information, and the setupfru (8) command specifies these locations and this information. The system administrator can use the commands for effective use of resources.

##### addfru / deletefru / replacefru

The addfru (8) command is used to select a device, such as a system board, fan unit, PSU, IO-BOX, or XSCF Unit, to add it to the server, and the deletefru (8) and replacefru (8) commands are used to select and remove or replace, respectively, such a device mounted in the server. Each type of operation can be performed interactively with menus.

##### showboards / addboard / deleteboard / moveboard

The showboards (8) command displays status information about a system board (XSB). The system administrator can use the command to find out whether a system board has been configured to a domain or unconfigured from it, and to find out whether this operation was successful. The addboard (8) command adds a system board to the domain, the deleteboard (8) command removes a system board, and the moveboard (8) command moves a system board.

##### showdomainmode / setdomainmode

In a certain domain, the user may want to suppress the break signal or panic with host watchdog or disable the automatic boot function. The system administrator can use the showdomainmode (8) command to display the related function settings and the setdomainmode (8) command to suppress or disable one of these functions for a domain.

## 5.6 View and Archive the XSCF Logs

This section describes the XSCF commands that fetch and display server operation logs, console logs, temperature histories, and error logs from XSCF log files which also configure the information for archiving XSCF logs to a host.

For details on error logs, see Appendix B.

- showlogs

- showarchiving / setarchiving

##### showlogs

The showlogs (8) command displays error logs, power logs, event logs, console logs, panic logs, IPL logs, and temperature/humidity logs. The system administrator can use the command to check the operating status of the server and the cause of any error in the system.

##### showarchiving / setarchiving

The showarchiving (8) and setarchiving (8) commands display and specify, respectively, the information required for saving XSCF log information to servers. The system administrator can use these commands to set up automatic, secure archiving of logs to a specified archive host.

## 5.7 User Management and Security Commands

This section describes the typical XSCF commands for user management and security management.

- showuser / adduser / deleteuser / enableuser / disableuser

- password / setprivilege / showpasswordpolicy / setpasswordpolicy

- showlookup / setlookup / showldap / setldap

- showaudit / setaudit / viewaudit

##### showuser / adduser / deleteuser / enableuser / disableuser

The showuser (8) command can be used to list XSCF user accounts or display information about a particular user account. The adduser (8) and deleteuser (8) commands add and delete user accounts. The enableuser (8) and disableuser (8) commands enable and disable, respectively, user accounts.

##### password / setprivilege / showpasswordpolicy / setpasswordpolicy

The password (8) and setprivilege (8) commands set passwords and user privileges, respectively, for user accounts. The showpasswordpolicy (8) and setpasswordpolicy (8) commands display and specify the validity of passwords and other password policy information.

##### showlookup / setlookup / showldap / setldap

The showlookup (8) and setlookup (8) commands display and specify information on whether an LDAP server should be used for looking up the authentication and the user privilege. The showldap (8) and setldap (8) commands display and specify LDAP client settings, which are used when retrieving data from an LDAP server.

##### showaudit / setaudit / viewaudit

The showaudit (8) and setaudit (8) commands display and specify information such as which events can be subject for auditing. The system administrator can use the viewaudit (8) command to display audit records (audit trail).

## 5.8 Use the XSCF Other Commands

The following XSCF Shell commands end the XSCF Shell and display version information. (Note)

- exit

- version

Note - The server provides many other commands. For details on these commands, see the man page or the XSCF Reference Manual.

##### exit

The exit (8) command ends the XSCF Shell.

##### version

The version (8) command displays the comprehensive firmware version (XCP version, see Note) of the XSCF firmware and POST/OpenBoot PROM firmware. The system administrator can display version information when upgrading firmware.

Note - XCP: XSCF Control Package that includes the programs which control the hardware components making up a computer system.

## 5.9 View XSCF Shell Error Messages

TABLE 5-2 lists the typical messages from each XSCF Shell command.

| TABLE 5-2 Error Messages of XSCF Shell Commands |  |
| --- | --- |
| Message | Meaning |
| Invalid parameter. | An abnormal parameter error has occurred. |
| Operation failed. | Abnormal end. |
| Permission denied. | An execution authority error has occurred. |
| Operation not supported on this system. | Unsupported function. |
| Operation interrupted. | Processing interruption from user. |
| The current configuration does not support this operation. | Abnormal configuration. |
| A hardware error occurred. Please check the error log for details. | A hardware error has occurred. |
| An internal error has occurred. Please contact your system administrator. | An XSCF internal error has occurred. |

Note - The error message depends on the command. Therefore, you will occasionally see more messages.

| SPARC Enterprise Mx000 Servers XSCF User’s Guide | 819-6202-13 |  |
| --- | --- | --- |

Copyright © 2008, Sun Microsystems, Inc. All Rights Reserved.