Configurer la vitesse des pattes des cartes réseau sur un serveur SUN

1.1	Introduction	1

1.2	Configurer la vitesse des pattes par la commande « ndd »	1

1.3	Configurer le fichier /platform/sun4u/kernel/drv/<drv>.conf	2

1.4	Vérifier les vitesses des cartes:	5

1.5	Annexe1 : le fichier /platform/sun4u/kernel/drv/bge.conf	8

1.6	Annexe2 : le fichier /platform/sun4u/kernel/drv/ce.conf	8

1.7	Annexe 3 : Liste des principaux drivers sur Sun:	10

## Introduction

Il y a normalement 3 façons d’’initialiser les vitesses des pattes réseau sur nos serveurs :

- On positionne les vitesses et les « modes », par des commandes « ndd » inscrite dans le fichier « /etc/rc2.d/S25conf-ethernet.ksh »  _ qui est exécuté a) en fin de boot du serveur, b) ou à la main.

2) On configure les vitesses des pattes et les « modes », au niveau d’un fichier de configuration du driver la carte réseau (fichier ce.conf, bge.conf … présents dans le répertoire /platform/sun4u/kernel/drv), dépendant du type de la carte (ce, bge …), qui est lu au moment de l’initialisation de ces vitesses au début du boot (initialisation faite au niveau du noyau Unix).

3) Il existe une troisième méthode qui consiste à positionner les vitesses dans « /etc/system ».

Recommandations SUN :

Pour les interfaces « ce », « bge » et « dmfe », SUN recommande d'utiliser le fichier de configuration du driver en lui-même. Les vitesses des ports seront donc positionnés dès le boot du system (par exemple pour « ce », dans le fichier « /platform/sun4u/kernel/drv/ce.conf ») .

La troisième méthode consistant à positionner les vitesses dans « /etc/system ». n'est pas recommandée par SUN.

Configurer ces derniers fichiers, obliger de rebooter le serveur.

## Configurer la vitesse des pattes par la commande « ndd »

(par exemple au niveau du fichier « conf-ethernet.ksh »).

Exemple pour forcer l'interface ce6 en 100 full duplex sans auto négociation:

a) Positionner l’instance :

[su0967@root:/] ndd –set /dev/ce instance 6

b) Voir tous les paramètres existants pour le driver.

[su0967@root:/]  ndd –get /dev/ce ?

[su0967@root:/] ndd -get /dev/ce ?

adv_autoneg_cap               (read and write)

adv_1000fdx_cap               (read and write)

adv_1000hdx_cap               (read and write)

adv_100T4_cap                 (read and write)

adv_100fdx_cap                (read and write)

adv_100hdx_cap                (read and write)

adv_10fdx_cap                 (read and write)

adv_10hdx_cap                 (read and write)

adv_asmpause_cap              (read and write)

adv_pause_cap                 (read and write)

master_cfg_enable             (read and write)

master_cfg_value              (read and write)

use_int_xcvr                  (read and write)

enable_ipg0                   (read and write)

ipg0                          (read and write)

ipg1                          (read and write)

ipg2                          (read and write)

rx_intr_pkts                  (read and write)

rx_intr_time                  (read and write)

red_dv4to6k                   (read and write)

red_dv6to8k                   (read and write)

red_dv8to10k                  (read and write)

red_dv10to12k                 (read and write)

tx_dma_weight                 (read and write)

rx_dma_weight                 (read and write)

infinite_burst                (read and write)

disable_64bit                 (read and write)

accept_jumbo                  (read and write)

laggr_multistream             (read and write)

cc) exemple : forcer une interface ce 6 en 100full duplex :

# ndd –set  /dev/ce instance 6

# ndd –set /dev/ce adv_autoneg_cap 0

# adv_1000fdx_cap 0

# adv_1000hdx_cap 0

# adv_100T4_cap 0

# adv_100fdx_cap 1

# adv_100hdx_cap 0

# adv_10fdx_cap 0

Attention: Avec du Gigabit il faut toujours laisser l'auto négociation.

Note : souvent, la patte SVG (voire la patte CLI) sont en 1000 Mbps (Gigabit) avec autonegociation.

## Configurer le fichier /platform/sun4u/kernel/drv/<drv>.conf

Ce fichier « /platform/sun4u/kernel/drv/<drv>.conf » peut être le fichier :

/platform/sun4u/kernel/drv/dmfe.conf

ou /platform/sun4u/kernel/drv/ce.conf

Ou /platform/sun4u/kernel/drv/bge.conf

(Note : pour le fichier « bge.conf » voir paragraphe suivant 3.3).

Ce fichier contient les mêmes paramètres que ceux à positionner par la commande ndd.

Exemple0: positionner toutes les pattes de la carte « ce » à 1000 FD, autoneg=1.

#adv_autoneg_cap=1

#adv_1000fdx_cap=1

#adv_1000hdx_cap=0

#adv_100fdx_cap=0

#adv_100hdx_cap=0

#adv_10fdx_cap=0

#adv_10hdx_cap=0

Ces paramètres sont alors positionnés pour tous les ports (pattes) utilisant ce driver.

Exemple1: positionner le ce0 en 100 full duplex et le ce1 en 1000 full duplex avec autoneg :

Ici, il faut introduire des paramètres particulieers, dans le fichier de configuration :

Par exemple, positionner a) la patte ce0 en 100 full duplex et b) puis celle ce1 en 1000 full duplex avec autoneg, sur un serveur Sun, il faut :

A) d’abord extraire certaines informations utiles (chemins physiques) pour chaque patte dans le fichier « /etc/path_to_inst »  :

# grep ce /etc/path_to_inst

"/pci@9,700000/network@2" 0 "ce"     <= ligne contenant le path/chemin physique pour la patte ce0

"/pci@9,600000/network@1" 1 "ce"     <= ligne contenant le path/chemin physique pour la patte ce1

Autre exemple :

# grep ce /etc/path_to_inst

"/pci@8,700000/pci@2/network@0" 0 "ce"

"/pci@8,700000/pci@2/network@1" 1 "ce"

"/pci@8,700000/pci@2/network@2" 2 "ce"

"/pci@8,700000/pci@2/network@3" 3 "ce"

"/pci@8,700000/pci@3/network@0" 4 "ce"

"/pci@8,700000/pci@3/network@1" 5 "ce"

"/pci@8,700000/pci@3/network@2" 6 "ce"

"/pci@8,700000/pci@3/network@3" 7 "ce"

"/pci@9,700000/network@2" 8 "ce"

"/pci@9,600000/network@1" 9 "ce"

B) Puis dans le fichier « ce.conf », il faut créer les 2 lignes suivantes, dans l’exemple ci-après, définissant les paramètres (ou config) réseau des pattes ce0 et ce1.

La syntaxe de ces lignes est de la forme :

name="type-patte" parent="chemin_physique"  unit-address="numero_patte" ;

Avec type-patte = ce, bge, hme etc …

La partie en vert (un extrait du chemin physique) est à mettre après le mot clé « parent=" » et le nombre en rouge (le n° physique de la patte), est à mettre après le mot-clé « unit-address=" » comme ci-après :

name="ce" parent="/pci@9,700000" unit-address="2"

adv_autoneg_cap=0

adv_1000fdx_cap=0

adv_1000hdx_cap=0

adv_100fdx_cap=1

adv_100hdx_cap=0

adv_10fdx_cap=0

adv_10hdx_cap=0;

name="ce" parent="/pci@9,600000" unit-address="1"

adv_autoneg_cap=1

adv_1000fdx_cap=1

adv_1000hdx_cap=0

adv_100fdx_cap=0

adv_100hdx_cap=0

adv_10fdx_cap=0

adv_10hdx_cap=0;

Notes :

1) chaque ligne ou groupe de ligne ci-avant, définissant les paramètres réseau (vitesse, mode, …) d’une patte précise, dans le fichier  « /etc/path_to_inst », doit se terminer par un « ; » (un point virgule).

2) Sinon, voici un exemple de fichier « ce.conf », configurant des vitesses différentes et des modes différents pour 3 pattes (SVG, CLI, ADM), donné en annexe de ce document.

3) On peut aussi extraire les paramètres inclus dans la ligne commençant par « name= » par :

[su0321@root:/platform/sun4u/kernel/drv] grep ce /etc/driver_aliases

vnex "SUNW,sun4v-virtual-devices"

vnex "SUNW,virtual-devices"

usb_mid "usb,device"

ce "pci108e,abba"

ce "pci100b,35"

pcelx "pccard101,556"

pcelx "pccard101,574"

pcelx "pccard101,589"

pcelx "pccard101,562,0"

ou bien

# grep bge /etc/driver_aliases

bge "SUNW,bge"

bge "pci108e,1647"

bge "pci108e,1648"

bge "pci108e,16a7"

bge "pci108e,16a8"

bge "pci14e4,1645"

bge "pci14e4,1647"

bge "pci14e4,16a7"

bge "pci14e4,16c7"

bge "pci14e4,1648"

bge "pci14e4,16a8"

bge "pci14e4,1649"

bge "pciex14e4,1659"

bge "pci14e4,1668"

bge "pci14e4,1669"

bge "pci14e4,1677"

bge "pci14e4,1678"

bge "pci14e4,167d"

Ce qui donne :

# patte ce0 CLI 1Go

name="pci108e,abba" parent="/pci@8,700000/pci@2" unit-address="0"

adv_autoneg_cap=1

adv_1000fdx_cap=1

adv_1000hdx_cap=0

adv_100fdx_cap=0

adv_100hdx_cap=0

adv_10fdx_cap=0

adv_10hdx_cap=0

;

# patte ce1 ADM 100 FD

name="pci108e,abba" parent="/pci@8,700000/pci@2" unit-address="1"

adv_autoneg_cap=0

adv_1000fdx_cap=0

adv_1000hdx_cap=0

adv_100fdx_cap=1

adv_100hdx_cap=0

adv_10fdx_cap=0

adv_10hdx_cap=0

;

# patte ce8 SVG 100 FD

name="pci108e,abba" parent="/pci@9,700000" unit-address="2"

adv_autoneg_cap=0

adv_1000fdx_cap=0

adv_1000hdx_cap=0

adv_100fdx_cap=1

adv_100hdx_cap=0

adv_10fdx_cap=0

adv_10hdx_cap=0

;

Exemple2: forcer l'interface 2 en gigabit:

[su0967@root:/] grep ce /etc/path_to_inst

"/pci@1d,700000/pci@1/network@0" 0 "ce"

"/pci@1d,700000/pci@1/network@1" 1 "ce"

"/pci@1d,700000/pci@1/network@2" 2 "ce"

"/pci@1d,700000/pci@1/network@3" 3 "ce"

name="ce" parent="pci@1d,700000/pci@1" unit-address="2" adv_autoneg_cap=1 adv_1000fdx_cap=1 adv_1000hdx_cap=0 adv_100fdx_cap=0 adv_100hdx_cap=0 adv_10fdx_cap=0 adv_10hdx_cap=0;

## Vérifier les vitesses des cartes:

1) Il existe un script qui permet de vérifier les vitesses des cartes. Ce scripts se trouvent dans /opt/sfrsi/admin/scripts/nicstatus.ksh

Attention: ce script fonctionne parfaitement en Solaris 10. En revanche il n'est pas 100 fiable en Solaris 8. Par exemple pour les interfaces « ce », en Solaris 8, le full duplex est « taggé » par le chiffre 1, alors qu'en Solaris 10 c'est le chiffre 2.

2) Vous pouvez aussi récupérer les informations à l'aide des commandes ndd et kstat.

Exemple1 avec kstat :

# kstat hme | egrep 'name|link_speed|link_up'

Exemple2 avec ndd :

# On positionne l'instance 4

# /usr/sbin/ndd -set /dev/qfe instance 4

# Pour afficher les valeurs des parametres

# ndd -get /dev/hme link_status #=> valeurs posssibles : 1 = Up,  0 = Down

# ndd -get /dev/hme link_speed  #=> valeurs posssibles : 1 = 100, 0 = 10

# ndd -get /dev/hme link_mode   #=> valeurs posssibles : 1 = full,0 = half

(Par exemple, en solaris 10 : kstat donne 2 pour le full duplex et 1 pour le half duplex. Le « ndd -get » donne 1 pour le full duplex).

Note : kstat est plus recommandé pour les cartes de type bge ou hme.

3) en Solaris 10, vous pouvez utiliser la commande « dladm show-dev ».

Exemple de la remonté d'information d'une carte ce en Solaris 8 et 10:

[root@su0428:/] uname -a

SunOS su0428 5.10 Generic sun4u sparc SUNW,Sun-Fire-V490

[root@su0428:/] ndd -set /dev/ce instance 5

[root@su0428:/] ndd -get /dev/ce ?

?                             (read only)

instance                      (read and write)

adv_autoneg_cap               (read and write)

adv_1000fdx_cap               (read and write)

adv_1000hdx_cap               (read and write)

adv_100T4_cap                 (read and write)

adv_100fdx_cap                (read and write)

adv_100hdx_cap                (read and write)

adv_10fdx_cap                 (read and write)

adv_10hdx_cap                 (read and write)

adv_asmpause_cap              (read and write)

adv_pause_cap                 (read and write)

master_cfg_enable             (read and write)

master_cfg_value              (read and write)

use_int_xcvr                  (read and write)

enable_ipg0                   (read and write)

ipg0                          (read and write)

ipg1                          (read and write)

ipg2                          (read and write)

rx_intr_pkts                  (read and write)

rx_intr_time                  (read and write)

red_dv4to6k                   (read and write)

red_dv6to8k                   (read and write)

red_dv8to10k                  (read and write)

red_dv10to12k                 (read and write)

tx_dma_weight                 (read and write)

rx_dma_weight                 (read and write)

infinite_burst                (read and write)

disable_64bit                 (read and write)

accept_jumbo                  (read and write)

laggr_multistream             (read and write)

[root@su0428:/] kstat -p | grep ce5

ce:5:ce5:adv_cap_1000fdx        0

ce:5:ce5:adv_cap_1000hdx        0

ce:5:ce5:adv_cap_100T4  0

……….

ce:5:ce5:link_duplex    2

ce:5:ce5:link_pause     0

ce:5:ce5:link_speed     100

ce:5:ce5:link_up        1

……

ce:5:ce5:xcvr_addr      1

ce:5:ce5:xcvr_id        536894584

ce:5:ce5:xcvr_inits     1

ce:5:ce5:xcvr_inuse     1

[root@barbadine:/] uname -a

SunOS barbadine 5.8 Generic_117350-06 sun4u sparc SUNW,Sun-Fire

[root@barbadine:/] ndd -set /dev/ce instance 0

[root@barbadine:/] ndd -get /dev/ce link_status

1

[root@barbadine:/]  ndd -get /dev/ce link_mode

1

[root@barbadine:/]   ndd -get /dev/ce link_speed

1

[root@barbadine:/] kstat -p | grep ce0

ce:0:ce0:alignment_err  0

ce:0:ce0:brdcstrcv      131131418

ce:0:ce0:brdcstxmt      6439

ce:0:ce0:cap_1000fdx    0

ce:0:ce0:cap_1000hdx    0

ce:0:ce0:cap_100T4      0

ce:0:ce0:cap_100fdx     1

ce:0:ce0:cap_100hdx     1

ce:0:ce0:cap_10fdx      1

ce:0:ce0:cap_10hdx      1

ce:0:ce0:cap_asmpause   0

ce:0:ce0:cap_autoneg    1

ce:0:ce0:cap_pause      0

ce:0:ce0:class  net

ce:0:ce0:code_violations        0

ce:0:ce0:collisions     0

ce:0:ce0:crc_err        0

ce:0:ce0:crtime 488.05444544

ce:0:ce0:excessive_collisions   0

ce:0:ce0:first_collision        0

ce:0:ce0:ierrors        0

ce:0:ce0:ifspeed        100000000

ce:0:ce0:ipackets       778383725

ce:0:ce0:ipackets64     778383725

ce:0:ce0:ipackets_cpu00 281931829

ce:0:ce0:ipackets_cpu01 154621492

ce:0:ce0:ipackets_cpu02 170835589

ce:0:ce0:ipackets_cpu03 170994815

ce:0:ce0:late_collisions        0

ce:0:ce0:lb_mode        0

ce:0:ce0:length_err     0

ce:0:ce0:link_T4        0

ce:0:ce0:link_asmpause  0

ce:0:ce0:link_duplex    2

ce:0:ce0:link_pause     0

ce:0:ce0:link_speed     100

ce:0:ce0:link_up        1

[su0260@root:/platform/sun4u/kernel/drv] dladm show-dev

ce0             link: unknown   speed: 0     Mbps       duplex: unknown

ce1             link: unknown   speed: 1000  Mbps       duplex: full

lpfc0           link: unknown   speed: 0     Mbps       duplex: unknown

lpfc1           link: unknown   speed: 0     Mbps       duplex: unknown

ce2             link: unknown   speed: 100   Mbps       duplex: full

ce3             link: unknown   speed: 0     Mbps       duplex: unknown

ce4             link: unknown   speed: 0     Mbps       duplex: unknown

ce5             link: unknown   speed: 0     Mbps       duplex: unknown

ce6             link: unknown   speed: 100   Mbps       duplex: full

ce7             link: unknown   speed: 100   Mbps       duplex: full

lpfc2           link: unknown   speed: 0     Mbps       duplex: unknown

lpfc3           link: unknown   speed: 0     Mbps       duplex: unknown

Conclusion:

Les vitesses des interfaces est indépendantes des adresses positionnées ou des routages. En général Il faut utiliser la commande « ndd » pour positionner les vitesses et le vérifier.

## Annexe1 : le fichier /platform/sun4u/kernel/drv/bge.conf

Il se trouve dans le répertoire : /platform/sun4u/kernel/drv

Exemple1:

name="pci108e,abba" parent="/pci at 21c,700000/pci at 1" unit-address="0"

adv_autoneg_cap=0 adv_1000fdx_cap=0 adv_1000hdx_cap=0

adv_100fdx_cap=1 adv_100hdx_cap=0 adv_100T4_cap=0 adv_10fdx_cap=0

adv_10hdx_cap=0;

Exemple2:

# interface bge0

name="bge" parent="/pci at 1f,700000" unit-address="2"

adv_autoneg_cap=0 adv_1000fdx_cap=0 adv_1000hdx_cap=0

adv_100fdx_cap=1 adv_100hdx_cap=0 adv_100T4_cap=0 adv_10fdx_cap=0

adv_10hdx_cap=0;

# interface bge1

name="bge" parent="/pci at 1f,700000" unit-address="2,1"

adv_autoneg_cap=0 adv_1000fdx_cap=0 adv_1000hdx_cap=0

adv_100fdx_cap=1 adv_100hdx_cap=0 adv_100T4_cap=0 adv_10fdx_cap=0

adv_10hdx_cap=0;

# interface bge2

name="bge" parent="/pci at 1d,700000" unit-address="2"

adv_autoneg_cap=0 adv_1000fdx_cap=0 adv_1000hdx_cap=0

adv_100fdx_cap=1 adv_100hdx_cap=0 adv_100T4_cap=0 adv_10fdx_cap=0

adv_10hdx_cap=0;

# interface bge3

name="bge" parent="/pci at 1d,700000" unit-address="2,1"

adv_autoneg_cap=0 adv_1000fdx_cap=0 adv_1000hdx_cap=0

adv_100fdx_cap=1 adv_100hdx_cap=0 adv_100T4_cap=0 adv_10fdx_cap=0

adv_10hdx_cap=0;

avec :

$ grep bge /etc/path_to_inst

"/pci at 1f,700000/network at 2" 0 "bge"

"/pci at 1f,700000/network at 2,1" 1 "bge"

"/pci at 1d,700000/network at 2" 2 "bge"

"/pci at 1d,700000/network at 2,1" 3 "bge"

## Annexe2 : le fichier /platform/sun4u/kernel/drv/ce.conf

Il se trouve dans le répertoire : /platform/sun4u/kernel/drv

Exemple1:

# ce.conf

# GigaSwift (ce) driver configuration file

# Copyright (c) 2000 by Sun Microsystems, Inc.

# All rights reserved.

#

#ident  "@(#)ce.conf      1.0.5     041110 SMI ES-PTS-NET/Lor"

#

################################################################################

# This file is not needed for most installations !

#

# It MAY be needed for rare special customizations. In this case,

# copy the file and edit the following example to suit your needs.

################################################################################

# a mettre dans /platform/sun4u/kernel/drv/ce.conf

#

# Depending on your changes, this file overwrites the Sun[TM] recommended

# default values (To use the default values, you do not need this file).

# In particular, Sun's[TM] recommendation is to leave Ethernet Auto-negotiation

# ON at both link partners (this is the default). See Product Documentation and

# SunSolve Infodocs 41665, 72033 for proper use first.

#

# Most parameters must be set on both sides to have effect

# (e.g check also switch and remote partner(s) settings).

#

# To install:

# 1) On SPARC systems, copy ce.conf to /platform/sun4u/kernel/drv/ce.conf

#    On x86 Opteron systems, copy ce.conf to /kernel/drv/ce.conf

# 2) edit file to your needs. Default is to do nothing.

# 3) reboot the system

#

# For settings per interface, name, parent and unit-address

# must be specified. Obtain the hardware path from /etc/driver_aliases and

# /etc/path_to_inst as shown below (unit-address is NOT the instance !) :

#

# % grep ce /etc/driver_aliases

# ce "pci108e,abba"

#           |                       % grep ce /etc/path_to_inst

#           |                       "/pci@21c,700000/pci@1/network@0" 0 "ce"

#           |                               |                      |

#           V                               V                      V

# name="pci108e,abba" parent="/pci@21c,700000/pci@1" unit-address="0"

#

# after this, append the settings you want.

# Repeat for each interface to be configured. All other will be default.

# Don't forget the ";" after each section. Hardware path examples:

#

# name="pci108e,abba" parent="/pci@21c,700000/pci@1" unit-address="0"

# (GigaSwift NIC in Sun Fire 15000 hsPCI Board IO 16 PCI Slot 1)

#

# name="pci100b,35" parent="/pci@9,600000/pci@2/pci@0" unit-address="1"

# (2nd port of QGE NIC in Sun Fire V880 PCI Slot 7)

#

# name="pci108e,abba" parent="/pci@9,700000" unit-address="2"

# name="pci108e,abba" parent="/pci@9,600000" unit-address="1"

# (V480R onboard interfaces)

#

# For global settings, no hardware path needs to

# be specified, the settings are used for all ce interfaces.

################################################################################

#

# Select hardware path (not needed if all interfaces set the same)

#name="" parent="" unit-address=""

#

## example: forced 1000 Mbit/s, 802.3x Flow Control send and receive

# Speed/Mode values, set the desired to 1, all other to 0:

# les donnees ci-dessous sont fournies par le fichier '/etc/path_to_inst'

# patte ce1 CLI 1Go FD (maj de cette patte le 31/1/08)

# info extraite de la ligne : '"/ssm@0,0/pci@1a,700000/pci@1/network@1" 1 "ce" '

name="pci108e,abba" parent="/ssm@0,0/pci@1a,700000/pci@1" unit-address="1"

adv_autoneg_cap=1

adv_1000fdx_cap=1

adv_1000hdx_cap=0

adv_100fdx_cap=0

adv_100hdx_cap=0

adv_10fdx_cap=0

adv_10hdx_cap=0

;

## patte ce2 ADM 100 FD

# info extraite de la ligne : '"/ssm@0,0/pci@1b,600000/pci@1/network@0" 2 "ce"'

name="pci108e,abba" parent="/ssm@0,0/pci@1b,600000/pci@1" unit-address="0"

adv_autoneg_cap=0

adv_1000fdx_cap=0

adv_1000hdx_cap=0

adv_100fdx_cap=1

adv_100hdx_cap=0

adv_10fdx_cap=0

adv_10hdx_cap=0

;

## patte ce6 SVGTEMP 100 FD, patte temporaire cree a la demande de Mathieu GABORO (BT SAS) le 13/2/08

# info extraite de la ligne : '"/ssm@0,0/pci@1e,700000/pci@1/network@0" 6 "ce"'

name="pci108e,abba" parent="/ssm@0,0/pci@1e,700000/pci@1" unit-address="0"

adv_autoneg_cap=0

adv_1000fdx_cap=0

adv_1000hdx_cap=0

adv_100fdx_cap=1

adv_100hdx_cap=0

adv_10fdx_cap=0

adv_10hdx_cap=0

;

## patte ce7 SVG 100 FD => passe a 1000FD (1Go) a la demande de Raouf (BT Reseau)

# le 7/2/2008, on remet a 100FD (BL)

# info extraites de la ligne : '"/ssm@0,0/pci@1e,700000/pci@1/network@1" 7 "ce"'

name="pci108e,abba" parent="/ssm@0,0/pci@1e,700000/pci@1" unit-address="1"

adv_autoneg_cap=0

adv_1000fdx_cap=0

adv_1000hdx_cap=0

adv_100fdx_cap=1

adv_100hdx_cap=0

adv_10fdx_cap=0

adv_10hdx_cap=0

;

#

## Enable Ethernet Flow Control

#adv_asmpause_cap=0

#adv_pause_cap=1

#

# Clock Master values, change in forced 1000 Mbit/s back-to-back config only:

#master_cfg_enable=1 # enable usage of master_cfg_value (ce >1.118)

#master_cfg_value=0 # one link partner 1, the other 0 (ce >1.118)

#

# Advertize values above per Auto-negotiation.

# Should be set to 1 in most cases, 0 disables Auto-negotiation

adv_autoneg_cap=0

#

#accept-jumbo=1

## Annexe 3 : Liste des principaux drivers sur Sun:

| Type | Description | Configuration |  |
| --- | --- | --- | --- |
| * hme | - FEPS chip and 10/100baseT fast Ethernet | #ndd -set /dev/hme instance 0 |  |
| * qfe | - Sun Quad FastEthernet | #ndd -set /dev/qfe instance 0 |  |
| * eri | - Sun Ethernet Rio Interface 10/100 Base. | #ndd -set /dev/eri instance 0 |  |
| * ce | - GigaSwift Gigabit Ethernet 1.x | #ndd -set /dev/ce instance 0 |  |
| * vge | - Gigabit Ethernet 1.x , | #ndd -set /dev/vge instance 0 |  |
| * ge | - Gigabit Ethernet 2.x/3.x | #ndd -set /dev/ge instance 0 |  |
| * gem | - Gigabit Ethernet MAC 1000 Base | #ndd -set /dev/ge instance 0 |  |
| * bge | - The fourth generation of Sun Gigabit Ethernet products | #ndd -set /dev/bge0 adv_1000fdx_cap 1 |  |
| * dmfe | - Davicom 10/100Mb Ethernet Drivers(use DM9102A Ethernet chipset) | #ndd -set /dev/dmfe0 adv_100fdx_cap 1 |  |
| * e1000g | - Intel Gigabit and 82546EB based 1000-baseT driver | ndd /dev/e1000g0 link_status |  |
| * nge | - Gigabit Ethernet Driver in Solaris 10 x86 | ndd -set /dev/nge0 adv_100fdx_cap 1 |  |

Ce fichier est, en fait, un lien sur le fichier « /etc/init.d/conf-ethernet.ksh ».

Ce point est valable pour Solaris 8 et Solaris 10.