## General Information

Source: Unknown

A biogas digester is an ecosanitation option. It uses human and animal feces and organic waste to produce gas that can be used in the home for cooking and lighting. A pour flush latrine can be connected to the biogas digester. The digester uses natural decomposition processes to create methane and carbon dioxide gas. The slurry produced has a high level of pathogens and should be composted and used as fertilizer or safely disposed (e.g. buried).  An average family cannot normally produce enough excreta to fuel a biogas digester on their own, and must add the feces of two cattle (or similar quantity livestock manure), or combine with other families.

## Recommended Areas

- Rural areas for households with livestock

- Compounds/communities with more than one family or household who own at least two cattle  who could share a digester

- Areas where fuel for cooking/lighting is expensive or in short supply

## Materials

- Requires skilled labour and advice

- Appliances that can be run off biogas: cooking stove, heating, lighting, refrigeration

## Design Components

- Masonry or steel digestion chamber

- Inlet port for loading animal excreta

- Connection to water seal / pour flush latrine with inlet port for human excreta (optional)

- Outlet for digested slurry

- Outlet port for generated gas

## Design Options

- A single biogas digester can be shared by a group of families to reduce capital costs. The arrangements for the operation and management of the digester and the sharing of the gas produced must be agreed.

- Biogas digesters can be connected to communal latrines where sufficient human excreta can be collected.

## Operation and Maintenance

- Operation and maintenance of latrine according to type connected

- The digester should be loaded on a daily basis to operate effectively

- Kitchen and garden waste (organic matter) can be added to the digester

- Non-organic and solid materials should not be put into the digester The slurry produced should be composted to reduce pathogens or safely disposed (e.g. buried)

| Advantages | Limitations |
| --- | --- |
| - Excreta is used as a resource to produce fuel - Permanent system - The slurry can be composted for use on crops - Can be used by washers and wipers who use soft materials for anal cleansing - Can be connected to a pour flush latrine - Reduces potential for groundwater contamination if slurry is properly disposed | - Expensive to build - Required skilled design and construction labour - Requires regular maintenance and feeding of the system - Must have enough excreta for the system to function - Slurry must be handled carefully and composted as it is not sanitary - Requires water for flushing - May be unacceptable to use excreta for this purpose – especially human excreta |